VEDIT - Visual Editor                                         Page 18

Command Mode - Properties

In command mode  all character  output goes  to the  current CP/M

console output device, which may or may not be the same screen as used

for the visual mode.  The user enters  command lines, which consist of

single  commands,  strings  of  commands  or  iteration  macros.  Each

command line is ended with an  [ESC] [ESC], at which point the command

line is executed.  The [ESC] is also used to delimit search strings.

Each character typed is echoed by VEDIT and none are processed by

CP/M.  Thus the [CTRL-C] has a different meaning in VEDIT and does not

cause a return to CP/M.  The [ESC], which may also be entered with the

[ALT MODE] key, is  echoed  with  a  "$"  which  is  also  used in the

examples in this  manual to  signify the  [ESC] key.  The  [RETURN] or

[CR] key is echoed with a [CR] [LF] pair, and the pair is also entered

into the command line.  Although this causes a new line to be printed,

it is still  part of  the command  line and  does not  end the command

line.

The user is prompted for a new command line by the "*" character.

If, while typing, the command line should exhaust the amount of memory

space available to  it, (the  text buffer,  text register  and command

line all share the  same  memory  space)  VEDIT  will  send the "Bell"

character to  the  console  and  neither  accept  nor  echo  any  more

characters.  The user will then have to  edit the current command line

in order to end  it and then rectify  the full memory situation.  Even

when the memory is full, (see "U" command) up to ten characters may be

typed on the command line.

Before the command line  is ended and  begins executing, the line

may be edited  with  most  common  line  editing characters.  They are

described  in  detail  below  under  "Line  Editing".  Once  execution

begins, it may often  be  aborted  by  typing  the [CTRL-C] character.

This causes a *BREAK* to  be printed and a  new command mode prefix to

be printed.  VEDIT checks for  the [CTRL-C] before  any new command is

executed and also during the  execution of the "A",  "F", "N", and "T"

commands, and in a few other situations.

A useful feature for  some search  operations is  the special "|"

character.  Each "|" in  the  string  being  searched  will  match any

character in the text.  Thus the search string "C|N" will match "CAN",

"C1N", "C N" and others.  Similarly, "C||E"  will match "CONE", "C NE"

and others.

(Please note  that  the  bracket  characters  used  for iteration

macros are printed as "["  and "]" in this  manual.  Some users may be

more  familiar  with  the  angle   brackets  " "  and  " ".  The  user

determines which characters to use during the customization process.)

VEDIT - Visual Editor                                         Page 19

Brief Command Description

'n' denotes a positive number.

'm' denotes a number which may  be negative to denote backwards

in the text buffer.

'string', 's1' and  's2' denote  strings which  may include the

[RETURN] key in  them.  'string' and 's1'  may also include

the "wildcard" character "|", each  of which will match any

character during the search.

'file' is a disk file name  in normal CP/M format with optional

disk drive and extension.

nA               Append 'n' lines from  the input  file to  the end of

the text buffer.  "0A" performs an auto-read.

B                Move the edit  pointer to  the beginning  of the text

buffer.

mC               Move the edit pointer by 'm' positions.

mD               Delete 'm' characters from the text.

E                First letter of extended two letter commands.

nFstring[ESC]    Search for the  'n'th  occurrence of  'string' in the

current text buffer  and  position  the  edit pointer

after it.

G                Insert the contents of the  text register at the edit

pointer.

Itext[ESC]       Insert the 'text'  into the  text buffer  at the edit

pointer.

mK               Kill 'm' lines.

mL               Move the edit pointer  by 'm' lines  and leave at the

beginning of that line.

nNstring[ESC]    Search for the 'n'th occurrence  of 'string' and read

more of the  file from  disk if  necessary.  The edit

pointer is positioned  after last  'string' if found,

else not moved or  left at the  beginning of the text

buffer.

mP               Put 'm' lines  of text into  the text register.  "0P"

empties the text register.

Ss1[ESC]s2[ESC]  Search for the  next  occurrence  of  's1' within the

current text buffer, and if found, change to 's2'.

VEDIT - Visual Editor                                         Page 20

Brief Command Description

mT               Print (type)  'm' lines.

U                Print # of  free bytes  remaining /  # bytes  in text

buffer/ # bytes in text register.

V                Go  into  visual  mode.  Set   cursor  position  from

current edit pointer.

nW               Write 'n' lines to the disk from the beginning of the

text buffer  and  delete  from  the  text buffer.  0W

writes out the text buffer up to the current line.

Z                Move the edit  pointer to  the last  character in the

text buffer.

VEDIT - Visual Editor                                         Page 21

Brief Command Description

EXTENDED COMMANDS

EA               Restart the editor  by completely  writing the output

file, closing it,  and  then opening  the output file

again  with   an  EB.  The   text  register   is  not

disturbed.

EBfile           Open the file "file" for both Read and Write and then

perform an  auto-read if  the input  file exists.  If

the file  does  not  exist,  "NEW  FILE"  is printed.

Gives error if an output file is still open.

EF               Close the current output file.

EQ               Quit the edit session and leave disk files exactly as

before the session started.

ERfile           Open the file "file"  for input. Gives  error if file

does not exist.

ES n k           Change the  value  of  switch  "n"  to "k". Currently

there are 4 switches:

1      Expand Tab with spaces         (0=NO 1=YES)

2      Auto buffering in visual mode  (0=NO 1=YES)

3      Start in visual mode           (0=NO 1=YES)

4      Point past text reg. insert    (0=NO 1=YES)

5      Cursor type                    (0, 1 or 2)

6      Cursor blink rate              (10 - 100)

7      Lower case convert             (0, 1 or 2)

8      Conditional convert character  (32 - 126)

ET               Set new tab positions.  The  ET is followed  by up to

30 decimal  numbers  specifying  the  tab  positions.

Since the positions start at  1, the normal positions

would be: 9 17 25 33 etc.

EV               Print the VEDIT version number.

EWfile           Open the file  "file" for  output.  Any existing file

by that name will be renamed to file.BAK following an

EF or EX.  Gives error  if an output  file is already

open.

EX               Exit back to  CP/M  after  writing  the  text and any

unappended part of the input file to the output file.

Gives error if no output file is open.

VEDIT - Visual Editor                                         Page 22

Command Mode - Detailed Command Description

nA     Append

--     ------

Example:       100A$$          0A$$

Description:   This command will append 'n'  lines from the input file

to the  end of  the text  buffer.  Fewer lines  will be

appended if there is insufficient  memory space for 'n'

lines, or there  are  not  'n'  lines  remaining in the

input file.  If 'n'  is 0,  an auto-read  is performed,

which reads all  of the  input file  or until  the main

memory is almost full.  The command can be issued (with

'n' not zero) after an auto-read to read in more of the

file.  An error is given if there is no input file open

when this command  is  issued.  The  input  file can be

opened with the EB  and ER  commands, or  when VEDIT is

invoked from CP/M.

Notes:         No indication  is given  if fewer  than 'n'  lines were

appended.  Use the "U"  command to see  if anything was

appended.  If the text  buffer is  completely full, the

text register cannot be  used and visual  mode will not

work well.

See Also:      Commands: EB, ER, U, W

Auto-Read

Examples:      ERTEXT.DOC$$

0A$$            The file 'TEXT.DOC'  is opened  and all

of the file  is read  in, or  until the

memory is almost full.

VEDIT - Visual Editor                                         Page 23

Command Mode - Detailed Command Description

B     Beginning

-     ---------

Example:       B$$

Description:   This command moves the edit pointer to the beginning of

the text buffer.  The beginning of the text buffer will

not be the beginning of the  text file if a "W" command

or an auto-write was done.  In  this case, use the "EA"

command to move back to the beginning of the text file.

Notes:

See Also:      Commands: EA, Z

Examples:      B12T$$          Moves the edit pointer to the beginning

of the text buffer and prints the first

12 lines.

mC     Change

--     ------

Example:       12C$$           -4C$$

Description:   This command moves  the edit  pointer by  'm' character

positions, forwards if 'm' is positive and backwards if

'm' is  negative.  The  edit  pointer  cannot  be moved

beyond the beginning or the end of the text buffer, and

an attempt to do so will  leave the edit pointer at the

beginning or the end respectively.  Remember that every

line normally ends  in  a  [CR] [LF]  (carriage return,

line feed), which represents two character positions.

Notes:

See Also:      Commands: D, L

Examples:      Fhello$-5C$$    Searches for the  word "hello",  and if

it is found, positions the edit pointer

at the beginning of the word.

VEDIT - Visual Editor                                         Page 24

Command Mode - Detailed Command Description

mD     Delete

--     ------

Example:       12D$$           -4D$$

Description:   This command  deletes  'm'  characters  from  the  text

buffer, starting at  the current  edit pointer.  If 'm'

is positive, the  'm'  characters  immeditately  at and

following the  edit  pointer  are  deleted.  If  'm' is

negative, the 'm' characters preceding the edit pointer

are deleted.  Fewer than 'm' characters will be deleted

if the ends of the text buffer are reached.

Notes:

See Also:      Commands: C, K

Examples:      100<FBIKES$-D$>$$   The 'S' will be  deleted from up to

100 occurrences of the word 'BIKES'.

E     Extended Commands

-     -----------------

Example:       EX$$            EV$$

Description:   This is not a  command  by  itself  but  just the first

letter of all the extended commands.

Notes:         No error is given if just E$$ is given.

See Also:      Extended commands.

Examples:

VEDIT - Visual Editor                                         Page 25

Command Mode - Detailed Command Description

nFs1[ESC]     Find

---------     ----

Example:       Fmispell$$      10Fwords$$

Description:   This command searches  the text  buffer, beginning from

the current edit  pointer, for the  'n'th occurrence of

the string 's1'.  The edit  pointer will  be positioned

after the last  character  of  the  'n'th occurrence of

's1' if it is  found.  If the 'n'th  occurrence of 's1'

is not found,  an error  will be  printed and  the edit

pointer will be positioned after the last occurrence of

's1' found, or be  left at its  original position if no

occurrences of 's1'  were found.  If  the search  is to

include parts of the  file not yet  in the text buffer,

use the "N" command.

Notes:         The search   is   always   forward,   never  backwards.

Remember that the "wild card" character can be used.

See Also:      Command: N

Examples:      BFhello$$       Search for the  word  "hello"  from the

beginning of the text buffer.

1000<3Ffirst$-5DIthird$>$$  Change     every      third

occurrence  of  the   word  "first"  to

"third".

Z-100LFend$$    Find the word  "end" if  it occurres in

the last 100 lines of the text buffer.

1000<Ffix up$V>$$   Finds the  next  occurrence  of the

string "fix up"  and  goes  into Visual

mode. Any changes can be made in Visual

mode.  When Visual mode  is exited, the

next occurrence of  "fix  it"  is found

and so on.

VEDIT - Visual Editor                                         Page 26

Command Mode - Detailed Command Description

G     Get

-     ---

Example:       G$$

Description:   This command inserts a copy of the text register at the

current edit pointer.  If there  is insufficient memory

space for the entire  copy, nothing is  inserted and an

error message is given.  If the text register is empty,

nothing is inserted.  The contents of the text register

are not affected  by this command.  The  "P" command or

visual mode is used to place text in the text register.

Notes:

See Also:      Commands: P

Visual Mode text move.

Examples:      BG$$            Inserts  the   contents  of   the  text

register at the  very beginning  of the

text buffer.

12<G>$$         Inserts  the   contents  of   the  text

register twelve  times  at  the current

edit pointer.

132P132K$$

EA$$

10LG$$          Moves 132 lines  of text,  by saving it

in  the  text   register,  killing  the

original lines  and inserting  the text

after the  tenth line  of the  file, in

the situation  where  the  beginning of

the file  is  no  longer  in  the  text

buffer.

VEDIT - Visual Editor                                         Page 27

Command Mode - Detailed Command Description

Itext[ESC]     Insert

----------     ------

Example:       Ia word$$       I[RETURN]new line$$

Description:   This command inserts  the  text  'text'  into  the text

buffer, starting  at  the  current  edit  pointer.  The

insertion is  complete  when  the  [ESC]  character  is

encountered.  The inserted text does  not overwrite any

existing text.  The  'text'  may  contain  the [RETURN]

key, which is expanded to  carriage return - line feed.

If insufficient memory  space exits for  the 'text', an

error will be printed and only  part of the 'text' will

have been inserted.  The  edit  pointer  is  moved just

past the inserted text.  This  command is probably best

used in iteration  macros, since  normal text insertion

is much easier to do in visual mode.

Notes:         The  character  [ESC]  cannot   be  inserted.  The  tab

character is not expanded with spaces as is optional in

visual mode.

See Also:

Examples:      200<I[CR][TAB]$>$$  Inserts   200   new   lines,   each

beginning with a tab character.

VEDIT - Visual Editor                                         Page 28

Command Mode - Detailed Command Description

mK     Kill

--     ----

Example:       4K$$            -3K$$         0K$$

Description:   This command  performs  a  line  oriented  deletion (or

killing) of text.  If  'm' is  positive, all characters

from the current edit  pointer and up  to and including

the 'm'th [LF]  are deleted  from the  text buffer.  If

'm' is  negative,  all  characters  preceding  the edit

pointer on the current line and the 'm' preceding lines

are deleted.  If 'm' is 0, all characters preceding the

edit pointer on  the  current  line  are deleted. Fewer

than 'm' lines will be killed if either end of the text

buffer is reached.

Notes:

See Also:      Command: D, T

Examples:      1000<Ftemp line$0LK>$$  Kills  all lines  which contain

the string "temp line".

-10000K$$       Kills all text before the edit pointer.

10000P10000K$$  Saves the rest  of  the  text  from the

edit pointer in  the text  register and

then deletes it from the text buffer.

VEDIT - Visual Editor                                         Page 29

Command Mode - Detailed Command Description

mL     Lines

--     -----

Example:       120L$$          -14L$$         0L$$

Description:   This command performs  a line oriented  movement of the

edit pointer, and  the edit  pointer is  always left at

the beginning of a line.  If  'm' is positive, the edit

pointer is left  following the  'm'th [LF].  If  'm' is

negative, the edit pointer is  left at the beginning of

the 'm'th  preceding  line.  If  'm'  is  0,  the  edit

pointer it moved to the  beginning of the current line.

Attempting to  move past  the ends  of the  text buffer

will leave the  edit  pointer  at  the  respective end.

This command makes no changes to the text buffer.

Notes:

See Also:      Commands: C, T

Examples:      1000<Stypo$type$0LT>$$  Changes  all   occurrences   of

"typo" to "type"  and prints  out every

line that was changed.

VEDIT - Visual Editor                                         Page 30

Command Mode - Detailed Command Description

nNs1[ESC]     Next

---------     ----

Example:       Nbad line$$     3Nthird$$

Description:   This command is very similar to the "F" command, except

that if the 'n'th  occurrence of  's1' is  not found in

the text buffer, auto-read/writes are performed to read

in more of the input file until the 'n'th occurrence is

found or the end of the  input file is reached.  If the

'n'th occurrence  still  is  not  found,  an  error  is

printed.  The edit  pointer  is  also  positioned  very

similar to  the "F"  command, except  in the  event the

'n'th occurrence is  not found and  neither the 'n-1'th

occurrence nor  the original  edit pointer  position is

any longer in the  text buffer.  In this  case the edit

pointer is positioned  at  the  beginning  of  the text

buffer.  Using this command  with  an  's1',  which you

know does not  exist, can  be used  to access  the last

part of a large file.

Notes:         The search is always forward, never backwards.

See Also:      Command: F

Auto Buffering

Examples:      1000<Ntypo$-4DItype$>$$ Changes all  occurrences of the

string "typo" to "type"  in the rest of

the file.

Nxcxc$$         Accesses the  last  part  of  the file,

assuming the string "xcxc" never occurs

in it.

VEDIT - Visual Editor                                         Page 31

Command Mode - Detailed Command Description

mP     Put

--     ---

Example:       40P$$           -20P$$         0P$$

Description:   This command saves a  copy of the  specified text lines

in the text  register.  The  previous  contents  of the

text register are destroyed.  The  range of lines saved

is the same as for the  "K" or "T" commands.  If 'm' is

zero, the text register is  simply emptied, and nothing

is saved  in it.  Since  the text  buffer and  the text

register share the  same memory  space, saving  text in

the  text  register  decreases  the  amount  of  memory

available to  the text  buffer.  Thus the  "0P" command

should be  given  when  the  saved  text  is  no longer

needed.  This command does not  change the text buffer.

If there is  insufficient  memory  space  for  the text

copy, the text  register  is  only  emptied, nothing is

saved in it and an error is printed.  The saved text is

inserted in the text buffer with  the "G" command or in

Visual mode.

Notes:         If the "P"  command occurs  within an  iteration macro,

the macro is aborted following the command.

See Also:      Commands: G, K, T

Visual Mode text move

Examples:      120P120K$$      The text  lines are  saved in  the text

register and then deleted from the text

buffer.

-23T$$

-23P$$          The  text   lines   are   printed   for

verification before  they are  saved in

the text register.

VEDIT - Visual Editor                                         Page 32

Command Mode - Detailed Command Description

Ss1[ESC]s2[ESC]     Substitute

---------------     ----------

Example:       Stypo$type$$

Description:   This command searches  for the next  occurrence of 's1'

in the text buffer and changes it to 's2'.  An error is

printed if 's1' is not found.  If there is insufficient

memory space for  inserting 's2',  's1' will  have been

changed to as much of 's2'  as possible and an error is

printed.  The edit pointer is positioned after 's2', if

's1' is found, or else is left at its original position

if 's1' is not  found.  See the "N"  command example on

how to perform a "substitute" if all of the file is not

in the text buffer.

Notes:         To completely kill a string  in an iteration macro, use

the command form: <Fs1$-nD$>$$

See Also:      Commands: F, N, I

Examples:      1000<Stypo$type$0LT>    Changes  all   occurrences   of

"typo" to "type"  and prints  out every

line that was changed.

1000<Sname$smith$V>$$   Change the  next  occurrence of

"name" to "smith"  and  go  into Visual

mode.  Any  changes  can   be  made  in

Visual mode  and  when  Visual  mode is

exited, the  next occurrence  of "name"

will be searched and so on.

VEDIT - Visual Editor                                         Page 33

Command Mode - Detailed Command Description

mT     Type

--     ----

Example:       14T$$           -6T$$         0T$$

Description:   This command prints  (types)  the  specified lines.  If

'm' is positive,  all characters from  the edit pointer

up to and including  the 'm'th [LF]  are typed.  If 'm'

is negative, the previous 'm'  lines and all characters

up to just preceding the  edit pointer are printed.  If

'm' is 0,  only  the  characters  on  the  present line

preceding the edit pointer are printed.  Fewer than 'm'

lines will be printed if either  end of the text buffer

is reached.  Note that  "0TT"  will  print  the current

line regardless of the position  of the edit pointer on

it.  This command does not move the edit pointer.  This

command is most useful in iteration macros for printing

selected lines.  Visual mode should be used for looking

at sections of a file.

Notes:

See Also:

Examples:      1000<Fmoney$0TT>$$  Prints out every  line in  the text

buffer with the string "money" in it.

U     Unused (Free Memory)

-     --------------------

Example:       U$$

Description:   This command prints the number of memory bytes free for

use by the  text buffer  or text  register, followed by

the number of  memory  bytes  used  by  the text buffer

(length of the text buffer),  followed by the number of

memory bytes used by  the text register  (length of the

text register).

Notes:         These three numbers will not always  add up to the same

total, since several  other  buffers  all  use the same

memory space.  If the  number of free  bytes goes below

260, the "FULL" flag will be set when in visual mode.

See Also:

Examples:

VEDIT - Visual Editor                                         Page 34

Command Mode - Detailed Command Description

V     Visual

-     ------

Example:       V$$

Description:   This command  enters  Visual  Mode.  The  visual cursor

position will be  set  from  the  current  edit pointer

position.  Visual  mode  is  exited   with  either  the

"Visual Exit"  or  the  "Visual  Escape" character.  At

that time the edit pointer will  be set from the cursor

position.

Notes:         The text register is preserved.

See Also:      Visual Mode

Examples:      Fhere$V$$       Find the word  "here" and  enter visual

mode.

nW     Write

--     -----

Example:       20W$$           10000W$$         0W$$

Description:   This command writes 'n' lines from the beginning of the

text buffer to the  output file and  then deletes these

lines from the text buffer.  If there are less than 'n'

lines in  the text  buffer, the  entire text  buffer is

written out and  deleted.  If 'n'  is zero,  the entire

text  buffer  up  to  the  line  the  edit  pointer  is

currently on,  is  written  out.  The  edit  pointer is

moved to the new  beginning of the  text buffer.  If no

output file is open, an error is printed and no text is

output nor deleted.  The output file can be opened with

an "EW" or "EB" command or when VEDIT is invoked.

Notes:         No indication is  given  if  less  than  'n' lines were

written.

See Also:      Commands: A, EB, EW, EX

Examples:      EWpart1.txt$$

24W$$

EF$$

EWpart2.txt$$

EX$$            The first 24  lines of  the text buffer

are written out to file "PART1.TXT" and

the rest of the  text buffer is written

out to file  "PART".TXT"  and  the edit

session is completed.

VEDIT - Visual Editor                                         Page 35

Command Mode - Detailed Command Description

Z     Zip

-     ---

Example:       Z$$

Description:   This  command  moves  the  edit  pointer  to  the  last

character in the text buffer.

Notes:         This command does not move the edit pointer to the last

character in the file  if the last part  of the file is

not yet in the text buffer.  See the "N" command on how

to bring the  last  part  of  the  file  into  the text

buffer.

See Also:      Commands: B, N

Example:       Z-100L$$        Positions the edit pointer to the 100th

line before the end of the text buffer.

Z-12T$$         Prints the  last  twelve  lines  in the

text buffer.

Nxcxc$Z-12T$$   Prints the  last  twelve  lines  in the

file, assuming the  string "xcxc" never

occurs in it.

VEDIT - Visual Editor                                         Page 36

Command Mode - Detailed Command Description

EA     Edit Again

--     ----------

Example:       EA$$

Description:   This command writes the  entire text buffer  out to the

output file, followed  by  the  remainder  of the input

file if  any  and  closes  the  output  file.  All file

backup and renaming  is performed  as with  the "EF" or

"EX" command.  The output file is then reopened as both

the input and output file and an auto-read on the input

file is performed.  This command thus starts a new edit

session and  is  functionally  equivalent  to  an  "EX"

command followed by invoking VEDIT  again with the name

of the current output file.  This  command has two main

purposes.  First,  it  acts  a  method  of  saving  the

currently edited file  on disk  as a  safeguard against

losing the file  due  to  a  user  error,  or hardware,

software or power failure.  Second, it acts as a method

of accessing the beginning of a large file after it has

been written out  to disk.  This is  especially true in

the case a block of  text is to be  moved from the rear

of a large file to the front, since the contents of the

text register are not affected by the "EA" command.  If

the "Start in  Visual Mode"  switch is  set, the editor

will go into visual mode following the "EA" command.

Notes:         Any commands following  the  "EA"  on  the command line

will be ignored, since the command line is cleared.

See Also:      Commands: B, G, EX

Visual Restart

Example:

132P132K$$

EA$$

10LG$$          Moves 132 lines  of text,  by saving it

in  the  text   register,  killing  the

original lines  and inserting  the text

after the  tenth line  of the  file, in

the situation  where  the  beginning of

the file  is  no  longer  in  the  text

buffer.

VEDIT - Visual Editor                                         Page 37

Command Mode - Detailed Command Description

EBfile[ESC]     Edit Backup

-----------     -----------

Example:       EBfile.txt$$

Description:   This command opens the  file 'file' for  both input and

output and then performs an  auto-read on the file.  It

is similar to the sequence of commands:

ERfile[ESC]EWfile[ESC]0A$$

except that if the file does not yet exist on disk, the

message "NEW FILE"  is printed.  If  an output  file is

still open, an error is printed  and the command has no

other effect.

Notes:         The term "backup" is used here to describe this command

since the term is used by some other editors to perform

a  similar   operation.  Remember  that   VEDIT  always

creates a "backup"  of a file  on disk, if  its name is

used as the name of the output file.

See Also:      Commands: ER, EW

Example:       10000W$EF$$

EBnewfile.txt$$ The entire  text buffer  is written out

to the current  output file,  that file

is closed,  and the  file "NEWFILE.TXT"

is opened for input and output and read

in.

ERpart1.txt$0A$$

EBpart2.txt$$   The file "PART1.TXT"  is read  into the

text buffer,  the  file  "PART2.TXT" is

then made the current  input and output

file and is appended to  the end of the

previous file "PART1.TXT".

VEDIT - Visual Editor                                         Page 38

Command Mode - Detailed Command Description

EF     Edit Finish (Close)

--     -------------------

Example:       EF$$

Description:   This command  closes the  output file  and the  file is

saved on disk.  No file is  saved on disk before either

this command or an "EX"  command is executed.  A backup

of any existing file on disk  with the same name as the

output file is  created  by  renaming  it  with  a file

extension of ".BAK".

Notes:         Since the output file is  actually opened with the CP/M

file extension ".$$$",  the .$$$ file  is first closed,

then any existing file  on disk  with the  same name as

the output file is renamed to  .BAK, and last, the .$$$

file is renamed to the true output file name.

See Also:      Commands: EW, EX

Example:       EWsave.txt$$

10000W$EF$$     The contents  of  the  text  buffer  is

written out as the  file "SAVE.TXT" and

that file is then closed.

VEDIT - Visual Editor                                         Page 39

Command Mode - Detailed Command Description

EQ     Edit Quit

--     ---------

Example:       EQ$$

Description:   This command quits the edit session without writing out

the text buffer  or closing any  output file.  Its main

purpose to "quit" after one  has made a mistake editing

and it seems best to leave  everything on disk just the

way it was  before  this  edit  session  began.  DO NOT

confuse this  command  with  the  "EA"  command;  their

results are  quite  opposite.  Remember  that  the "EA"

command starts a new edit session.

Notes:         Any output file  with  the  file  extension ".$$$" will

also be deleted.  Any  original file  on disk  with the

same name as the output file,  but with an extension of

".BAK"  will  have  been  deleted   if  more  than  128

characters were written  to  the  (now  deleted) output

file.  With the exception of this possible backup file,

all other files  will exist  on disk  just as  they did

before the aborted edit session.

See Also:      Commands: EA

Example:       1000K$$         Shoot!! Meant -1000K$$

EQ$$            Since a bad  mistake  was  made  in the

above command, it is best to abort this

edit session, go back to CP/M and start

over.

VEDIT - Visual Editor                                         Page 40

Command Mode - Detailed Command Description

ERfile[ESC]     Edit Read

-----------     ---------

Example:       ERnewfile.txt$$

Description:   This command opens the file 'file' for input (reading).

Nothing is read into the text buffer with this command.

The "A"  command or  an auto-read  is used  to actually

read the input file.  If the same file was already open

for input, the file is "rewound",  so that the file can

again be read from the  beginning.  An error is printed

if the file 'file' does not exist.

Notes:

See Also:      Commands: A, EB, EW

Example:       ERparts.inv$$

20A$$           The  file  "PARTS.INV"  is  opened  for

input and  twenty  lines  from  it  are

appended to the end of the text buffer.

ES n k[ESC]     Edit Set

-----------     --------

Example:       ES 1 0$$        ES 3 1$$

Description:   This command changes  the value  of switch  'n' to 'k'.

Currently  there  are  8   switches.  The  numbers  are

specified in decimal and separated by spaces or commas.

The default values  of  these  switches  are determined

during the customization process.  An error is given if

'n' is specified out of range.  The switches are:

1~~~~Expand Tab with spaces~~~~~~~~~(0=NO 1=YES)

2~~~~Auto buffering in visual mode~~(0=NO 1=YES)

3~~~~Start in visual mode~~~~~~~~~~~(0=NO 1=YES)

4~~~~Point past text reg. insert~~~~(0=NO 1=YES)

5~~~~Cursor type~~~~~~~~~~~~~~~~~~~~(0, 1 or 2)

6~~~~Cursor blink rate~~~~~~~~~~~~~~(10 - 100)

7~~~~Lower case convert~~~~~~~~~~~~~(0, 1 or 2)

8~~~~Conditional convert character~~(32 - 126)

Switch (1) determines  whether or not  the tab key

in visual mode is expanded with  spaces to the next tab

position.  If not, a tab character is inserted into the

text buffer.  Except for special  applications, the tab

key would not normally be expanded with spaces.

VEDIT - Visual Editor                                         Page 41

Command Mode - Detailed Command Description

Switch   (2)    determines    whether    or    not

auto-buffering is enabled in  visual mode.  The editing

of a large file is usually simpler with this switch on,

since  the  user   does  not  need   to  give  explicit

Read/Write commands.  If  some  more  complicated  file

handling, with explicit Read/Write commands (ER, EW, A,

W) is being done, the switch should then temporarily be

set off.

Switch (3)  determines  whether  or  not  the edit

session will  begin  in  visual  mode.   Changing  this

switch while running VEDIT will  only apply to the "EA"

command.

Switch (4) determines the  edit pointer's position

(or cursor's in  visual  mode)  following  insertion of

the text register.  If  the  switch  is  off,  the edit

pointer is not moved, and is thus left at the beginning

of the newly inserted  text.  If the switch  is on, the

edit pointer is  moved  just  past  the  newly inserted

text.

Switch (5) determines the type of cursor displayed

in visual mode in memory mapped versions of VEDIT.  The

CRT terminal versions use the terminals cursor instead.

The  cursor  types  are:  0=Underline,  1=Blink  Block,

2=Block.

Switch (6) determines the  cursor's blink rate for

cursor types 0 and 1 above.

Switch   (7)   determines   whether   lower   case

characters are converted to  upper case.  For value (0)

no conversion takes place,  for (1) all  lower case are

converted to  upper case,  and for  (2) lower  case are

converted to upper  case, unless  the cursor  is past a

"special" character on  the text  line.  This "special"

character  is  set  by  switch  (8).  All  of  this  is

primarily applicable to  assembly language programming,

where it  is desirable  to have  the Label,  Opcode and

Operand in upper  case  and  the  comment  in upper and

lower case.

Switch (8) sets  the conditional  upper/lower case

convert character used for switch (7) above.

Notes:         While the switch  values were  specified in hexadecimal

during customization, they must be specified in decimal

in command mode.

See Also:      Customization, Visual Mode

Example:       ES 1 1$$        This enables tabs  typed in visual mode

to be expanded with spaces.

VEDIT - Visual Editor                                         Page 42

Command Mode - Detailed Command Description

ET     Edit Tab

--     --------

Example:       ET 20 40 60 80 100 120$$

Description:   This command changes  the tab  table used  by VEDIT for

displaying tab characters, and in Visual mode, when the

"Expand  Tab"   switch  is   set,  for   expanding  tab

characters.  Up to 30  tab  positions  are  allowed and

they must  be  in  the  range  1  -  254.  The  default

positions  are  set   during  customization.  For  word

processing the tabs can be set to the same positions as

are specified for the text  formatting program in order

to see  how they  will look  in the  final product.  An

error is printed if a bad position is given.  No tab is

needed at position 1, and counting  starts at 1 (not at

zero).  Thus the normal tab positions would be:

9 17 25 33 41 49 57 65 73 81 89 97

105 113 121 129

Notes:         For use in Visual mode, there  must be at least one tab

position per screen line.

See Also:      Customization, Visual Mode

Example:

EV     Edit Version

--     ------------

Example:       EV$$

Description:   This command  prints  the  VEDIT  version number.  This

number should be  used in  any correspondence  you have

with  us  concerning  the   operation  of  VEDIT.  This

command can also  be  used  inside  iteration macros to

give some indication of the progress being made in long

executing macros.

Notes:

See Also:

Example:

VEDIT - Visual Editor                                         Page 43

Command Mode - Detailed Command Description

EWfile[ESC]     Edit Write

-----------     ----------

Example:       EWnewdat.inv$$

Description:   This command  opens  the  file  'file'  for  output and

subsequent writing.  No text is actually writen by this

command.  Some file must be opened  for output in order

to save any text on disk.  A file can also be opened by

the "EB", "EA" commands and  when VEDIT is invoked from

CP/M.  If a file is  already open for  output, an error

is printed and no other action takes place.

Notes:         The file opened is  actually a temporary  file with the

same name, but  with an extension  of ".$$$".  The file

is not made permanent and given  its true name until it

is closed with  the "EF",  "EA", or  "EX" commands.  At

that time, any existing file on disk with the same name

as the output file is backed  up by renaming it with an

extension of  ".BAK".  Any existing  file on  disk with

that name and the  .BAK extension will  be deleted when

more than 128 bytes  (the first sector)  are written to

the output file.

See Also:      Commands: W, EA, EF, EX

Example:       EWpart1.txt$$

24W$$

EF$$

EWpart2.txt$$

EX$$            The first 24  lines of  the text buffer

are written out to file "PART1.TXT" and

the rest of the  text buffer is written

out  to   file  "PART".TXT"   and  edit

session is completed.

VEDIT - Visual Editor                                         Page 44

Command Mode - Detailed Command Description

EX     Edit Exit

--     ---------

Example:       EX$$

Description:   This is  the  normal  exit  from  VEDIT  when  the file

currently being edited is to written out to disk.  This

command writes the entire text buffer out to the output

file, followed by  the remainder  of the  input file if

any, closes  the output  file and  exits back  to CP/M.

All file backup and  renaming is done  as with the "EF"

command.  An error is  printed  if  no  output  file is

open, and no other action is taken.

Notes:

See Also:      Commands: EB, EF, EW, EA, EQ

Example:       VEDIT FILE.TXT

V$$

EX$$            The editor is invoked in the normal way

to edit a file.  The  file is edited in

visual mode, and when  done, the normal

exit back to CP/M is made.

VEDIT - Visual Editor                                         Page 45

Command Mode - Line Editing

Several common control characters are  recognized in command mode

as line editing characters.  They are:

[CTRL-H] or [BACKSPACE] Delete the  last  character  typed  and echo a

[CTRL-H] to the console.

[RUBOUT] or [DELETE]    Delete the last  character typed  and echo the

deleted character to the console.

[CTRL-U]                Delete the entire command line  and send a "#"

to the console.

[CTRL-R]                Doesn't change the  command  line,  but echoes

the entire command line back to the console.

#############################################