BURROUGHS 9252 PRINTER SWITCH SETTINGS

Switches (0 = OFF, 1 = ON)

+++

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |                          |

+++

| 0 | 0 |   |   | 0 |   |   |   | USA ASCII Character Set  |

+++

| 0 | 0 |   |   | 1 |   |   |   | Germany Character Set    |

+++

| 0 | 1 |   |   | 0 |   |   |   | Canada Character Set     |

+++

| 0 | 1 |   |   | 1 |   |   |   | Finland Character Set    |

+++

|   |   | 0 |   |   |   |   |   | CR causes LF             |

+++

|   |   | 1 |   |   |   |   |   | CR does not cause LF     |

+++

|   |   |   |   |   | 1 |   |   | Skip over perforation    |

++                          |

|   |   |   |   |   |   |   |   | to Top Of Form when      |

++                          |

|   |   |   |   |   |   |   |   | printer is .75 inch of   |

++                          |

|   |   |   |   |   |   |   |   | perforation.             |

+++

|   |   |   |   |   | 0 |   |   | Do not skip over perf.   |

+++

|   |   |   |   |   |   | 1 |   | 17.2 Characters per Inch |

+++

|   |   |   |   |   |   | 0 |   | 10   Characters per Inch |

+++

9252 Printer Control Codes

Control	Hex	Dec		Description

Code _	_ _	_ _		_                                        _

BEL	07	07	Causes an audiable alarm to sound for approx. 1.5 seconds.

HT	09	09	Moves print head to the next Horizontal Tab (HT) position. These positions are a fixed 8 characters apart.

LF	0A	10	Advances the paper one line.

VT	0B	11	Advances paper to next Vetical Tab (VT) position. On power up VT positions are one inch apart, but can be changed under program control.

FF	0C	12	Advances paper to Top Of Form.

CR	0D	13	Used as a PRINT COMMAND causing that line of data to be printed. Can also cause an automatic line feed (LF) if Switch 3 is on.

SO	0E	14	Causes subsequent data up to andor CR codes or the End Of Line to print double width (5 CPI) characters.

SI	0F	15	Terminates extended character mode and causes a return to normal print.

DC2	12	18	Same as ESC 7 (force to condensed print 17.2 CPI)

9252 Printer Control Codes

Control	Hex	Dec		Description

Code _	_ _	_ _		_                                        _

ESC 0	1B,30	27,48	Makes current print line TOF.

ESC 1	1B,31	27,49	Clears all vertical tabs and sets page length to 24 inches.

ESC 2	1B,32	27,50	Enables 8 LPI Switch.

ESC 3	1B,33	27,51	Makes current print line a vertical tab position.

ESC 4	1B,34	27,52	Causes printer to space at 6 LPI until ESC 5, ESC 2 or RESET.

ESC 5	1B,35	27,53	Causes printer to space at 8 LPI until ESC 4, ESC 2 or RESET.

ESC 6	1B,36	27,54	Forces printer to print in normal print mode (10 CPI).

ESC 7	1B,37	27,55	Forces printer to print in condensed print mode (17.2 CPI).

ESC 8	1B,38	27,56	Puts printer into whichever print mode is set by switch 7.

ESC 6 SO

1B,36,0E	27,54,14	Puts printer into EXTENDED (5 CPI) print mode .

ESC 7 SO

1B,37,0E	27,55,14	Puts printer in CONDENSED or EXTENDED (8.6 CPI) mode.