DIP Switches on Operation Panel Circuit Board

DIP   |                       |                    |

SWITCH |          ON           |          OFF       |

SW1   | Character Sets        |  All switches off  |

SW2   |                       |  for USA character |

SW3   |                       |  set               |

SW4   |                       |                    |

SW5   | 7 Bits                | 8 Bits             |

SW6   | If CR is received,    | If CR is received, |

| printer does a        | printer does a CR. |

| CR  LF.              |                    |

SW7   | DEL code causes       | Printer ignores    |

| printerto print.      | DEL code.          |

SW8   | RS232 interface      | Parallel(Centronix)|

| enable (1200 baud max)| interface enable   |

Jumper Plugs SP1  SP2 on LEPV1 Board

SP1 used for RS232 (serial) interface only.

Side A = DTR is ON if printer is ready.

Side B = DTR is ON if power is on to printer.

SP2 used for Centronix (parallel) interface only.

Side A = Bit 8 supplied by host computer.

Side B = Bit 8 is grounded by printer.

DIP Switch for RS232 on LEPV1 Board

SWITCH |           ON          |          OFF          |

SW1   | RTS on                | RTS off               |

| when printer ready,   | when printer ready,   |

| off when printer      | on when printer       |

| not ready.            | not ready.            |

SW2   |                                               |

SW3   | Baud Rate see below                           |

SW4   |                                               |

SW5   | NOT USED                                      |

SW6   | Even Parity           | No parity             |

BAUD RATE

DIP Switch       |                                |

|          Baud Rate             |

SW2  |  SW3  |  SW4  |                                |

OFF  |  OFF  |  OFF  |               110              |

ON  |  OFF  |  OFF  |               150              |

OFF  |   ON  |  OFF  |               200              |

ON  |   ON  |  OFF  |               300              |

OFF  |  OFF  |   ON  |               600              |

ON  |  OFF  |   ON  |              1200              |

CONTROL CODES

Control

Code	Hex	Description

LF	0A	Moves paper up 1 line.

CR	0D	Returns Print head to column 1.

FF	0C	Feeds paper to first line of next pg.

VT,XX	0B,XX	Feeds paper to channel number XX.

XX = 31 to 3C.

DC1	11	Sets printer to online mode.

DC3	13	Sets printer to offline mode.

DC4	14	Loads tab position in VFU.

RS	1E	Sets printing to 10 CPI.

GS	1D	Sets printing to 16.5 CPI.

US	1F	Sets printing to 5 CPI.

CONTROL CODES

Control

Code	Hex	Description

ESC,

VT,XX	1B,0B,XX	Skips the number of lines designated

by XX. XX = 30,30 to 39,39.

ESC,

F,XX	1B,46,XX	Sets page length to number of lines

in XX. XX = 30,30 to 39,39.

ESC,5	1B,35	Sets current line to TOF.

ESC,6	1B,36	Sets spacing to 6 LPI.

ECS,8	1B,38	Sets spacing to 8 LPI.

ESC,A	1B,41	Designates long line

ESC,B	1B,42	Designates short line

SO	0E	Shift out character set (7 bit code)

SI	0F	Shift in character set (7 bit code)

CAN	18	Clears printer buffer.

9253 Printer Control Codes

||

| Command |      Function code      |      Description                  |

|         |  Decimal    Hexidecimal |                                   |

|	LF	10	0A	Moves paper up one line      |

|	CR	13	0D	Returns print head           |

|	FF	12	0C	Feeds paper to the first line|

|				of next format (TOF).        |

|	VT	11	0B	Feeds paper to tab position  |

|	Channel	49 to	31 to	of same channel number as set|

|	No.	60	3C	in VFU.                      |

|	DC1	17	11	Sets the printer in select   |

|				(online) condition.         |

|	DC3	19	13	Releases the printer from    |

|				select condition. and sets it|

|				in deselect (offline) cond. |

|	DC4	20	14	Loads tab position in VFU.   |

|	RS	30	1E	Designates 10 CPI            |

|	GS	29	1D	Designates 16.5 CPI          |

|	US	31	1F	Designates 5 CPI with RS     |

|                                          Designates 8.3 CPI with GS   |

|	ESC,VT	27,11	1B,0B	Directly skips the designated|

|	00	48,48	30,30	number of lines.             |

|	to 99	to 57,57	to 39,39	                             |

|	ESC,F	27,70	1B,46	Designates the number of     |

|	00	48,48	30,30	lines for page length.       |

|	to 99	to 57,57	to 39,39	                             |

|	ESC,5	27,53	1B,35	Sets TOF to Current Line     |

|	ESC,6	27,54	1B,36	Designates 6 LPI             |

|	ESC,8	27,56	1B,38	Designates 8 LPI             |

|	ESC,A	27,65	1B,41	Designates long line         |

|	ESC,B	27,66	1B,42	Designates short line        |

|	SO	14	0E	Shifts out character set in  |

|				case of 7bit code           |

|	SI	15	0F	Shifts in character set in   |

|				case of 7bit code           |

|	CAN	24	18	Clears print buffer          |