B25 video signals at the CPU. Monochrome  no graphics).

Pin	Assignment

1	Speaker

2	Power Enable

3	No connection

4	Monitor ID

5	Horizontal Sync Return

6	Vertical Sync Return

7	Video +

8	Halfbright +

9	Keyboard out return

10	Keyboard in return

11	+36VDC return

12	+36VDC power

13	+36VDC power

14	Keyboard Reset

15	No connection

16	No connection

17	Horizontal Sync +

18	Vertical Sync +

19	Video return

20	Halfbright return

21	Keyboard out +

22	Keyboard in

23	+36VDC return

24	+36VDC return

25	+36VDC return

Vertical Scan Rate     60Hz.

Horizontal Scan Rate   22.2 KHz.

Video Rate             20.1 Mbps

Resolution (Pixels)    720 Horizontal

348 Vertical

Display Format         29 rows

80 characters

Soft Font

B25 video sigals at the Graphics adapter. (Color or Monochrome bit mapped graphics).

Pin	Assignment

1	Speaker

2	Power Enable

3	Blue Video +

4	Blue Video

5	Horizontal Sync Return

6	Vertical Sync Return

7	Green Video +

8	Green Video

9	Keyboard out return

10	Keyboard in return

11	+36VDC return

12	+36VDC power

13	Not Used

14	Keyboard Reset

15	Red Video +

16	Red Video

17	Horizontal Sync +

18	Vertical Sync +

19	Return

20	Return

21	Keyboard out +

22	Keyboard in

23	Not Used

24	Not Used

25	Not Used