Digi Avanstar 8 xp PCI Multiport Board

Overview

Digi Avanstar supports Winframe 1.5 and higher with driver Ver 1.23.

•	8 or 16 programmable ports

•	Supports PCI bus type.

•	Surge protection on all signals

•	Fully shielded cable

•	20MHz RISC-based CD1400 UARTs

•	Supports up to 115.2Kbps, programmable

•	Software-selectable connection panel EIA-232and/or EIA-422

Requirements

Software Requirements

n	WinFrame, Version 1.5 or higher

n	WinFrame Client, Version 1.5 or higher

n	ACL / Avanstar Device Driver for Windows NT 3.5 & 3.51 Ver 1.23  diskette

Driver name : ACLSER.SYS	Size : 90664	Date : 03/06/96

Hardware Requirements

n	A WinFrame server

n	A Digi PCI Avanstar

Installation

Hardware Installation

- Turn the computer and all peripherals off.

- Remove the cover and install the board into an empty, compatible slot.

- Replace the cover and connect the external panel to the Avanstar board.

- Reconnect everything and turn the computer on.

If there are any problems, consult the manual.

Software Installation

In Winframe, open Control Panel from the Main group and run “Network.”

Click on “Add Adapter,” and select “<Other> Requires disk from manufacturer” and continue by inserting the ACL/Avanstar Windows NT driver diskette.

PCI Bus Adapter Installation

- Select “Avanstar Xp (100p, 16p, 8p)” from the “Board Type” drop-down list box.

- Select the number of ports that the adapter has.

The ACL / Avanstar device driver for Windows NT will only allow configuration of the port assignments for the PCI bus adapter.