HARDWARE FOR APPLEBUS - ETHERNET GATEWAY

Bill Croft and Nick Veizades

Stanford SUMEX project.

1. INTRODUCTION

The gateway is a multibus system consisting of 3 cards:

"SUN" CPU board, 8 or 10 MHz, 256K RAM

INTERLAN NI3210 10mb ethernet card.  This has onboard memory, so DMA

is not needed on the processor.  The Intel 82586 ethernet chip

does automatic buffer chaining of packets.

SUMEX applebus card.  This is described below;  it contains about

6 chips and can be wirewrapped in an afternoon.

2. SUMEX APPLEBUS CARD

2.1 PARTS

ARTEC (phone 415-592-2740) "multibus multipurpose interface board" ($165).

This board comes with the bus buffering and control logic all setup.

All you basically have to do is connect up your data IO lines and chip select.

ZYLOG Z8530 SCC; serial communications controller.

AMD 26LS32B; quad differential line receiver.

AMD 26LS30; dual differential tri-stateable driver.

DB-9, PC board mount, female.

7492; divide by six.

DIP pullup resistor pack (Beckman B98-1-R6.8K) 15 resistors.

XTAL, 22.1184 MHz; this is a "standard" frequency found in most stores.

Single package TTL oscillator, OR:

74S04; clock "oscillator".

2 1K resistors.

100 pf capacitor.

68 pf capacitor.

2.2 WIRING: OSCILLATOR

If you don't use a single package oscillator, this simple circuit

acts as one:  (you might run the output though a couple more spare

inverters in the S04 to further "clean it up")

----\/\-----       ----\/\-----

|    1K    |  68p  |    1K    |

|  |\      |       |   |\     |

|  | \     |  | |  |   | \    |

---|  \O---|--| |--|---|  \O--|----

|  | /        | |      | /    |

|  |/                  |/     |

|      100p         X         |

|    | |           |X| 22.1184|

|----| |-----------|X|--------|

| |           |X|

X

2.3 WIRING: 7492 (divide by six)

from		pin		to		pin

OSC 22.1184 MHz			B input		1

gnd				gnd		10

+5				Vcc		5

gnd				reset inputs	6,7

Qd output	8		SCC PCLK	20

The PCLK is derived as follows:  22.1184 MHz divided by 6 gives

3.67864 MHz;  this is connected to the SCC PCLK, RTxCA, and RTxCB.

In the digital phase lock loop mode, the SCC divides RTxC by 16

to get the FM0 rate:  3.67864 / 16 = 230400 bits per second.

2.4 WIRING: Z8530 SCC

from		pin		to		pin

gnd				gnd		31

+5				Vcc		9

7492 output	8		PCLK		20

PCLK		20		RTxCA,RTxCB	12,28

board DOUT	0 thru 7	board DIN	0 thru 7

board DOUT/IN	0		D0		40

board DOUT/IN	1		D1		1

board DOUT/IN	2		D2		39

board DOUT/IN	3		D3		2

board DOUT/IN	4		D4		38

board DOUT/IN	5		D5		3

board DOUT/IN	6		D6		37

board DOUT/IN	7		D7		4

pullup on each data line	D0-D7		1,2,3,4,37,38,39,40

board BRD\			board DOUTE\

board BRD\			RD\		36

board BWR\			board DINE\

board BWR\			WR\		35

board BEN\			CE\		33

pullup				INTACK\		8

pullup				IEI		7

INT\		5		board INT4\

board buffered INT4\		board multibus	37

board ADR0			A/B\		34

board ADR1			D/C\		32

board ADR enb\			gnd

TxDA		15		LS30 in A	2

RTSA\		17		LS30 enb B\	3

LS32B out A	3		RxDA		13

2.5 WIRING: 26LS32B receiver

from		pin		to		pin

DB9		8		A in +		2

DB9		9		A in -		1

A out		3		SCC RxDA	13

gnd				enable\		12

gnd				gnd		8

+5				Vcc		16

2.6 WIRING: 26LS30 driver

from		pin		to		pin

SCC TxDA	15		A in		2

SCC RTSA\	17		B enb\		3

out A		15		DB9		4

out B		14		DB9		5

gnd				mode		4

gnd				gnd		5

+5				Vcc		1

2.7 WIRING: other board straps and switches

S1 - IO MAP

S2 - 0110 0000 0000;  sets board address to 0x1f6000

S3 - all bits of address significant

ACK DELAY:  XACK to CCLK*6

Copyright (C) 1984 Stanford Univ. SUMEX project.

May be used but not sold without permission.