@(#)Doc 1.1 8/14/84 Copyright Sun Microsystems

120' Memory Diagnostic

O. Preface

1. purpose

2. audience

I. General Information

1. intro

diagnostic tool

motivation

flexibility

2. environment

this diag runs standalone

unix is not present within the system when diagnostic is running

hence the name standalone, this is necessary since unix

is rather picky about allowing free access to resources

all i/o support provided by the standalone libraries linked to the

diagnostic

3. features - usefulness to engineer, manufacturing, field

flexibility for engineers (in terms of sequencing and parameterization)

default parameterization and sequencing for manufacturing (ie hit one

key to start recommended test auto sequencing and default

parameters)

both for the field (includes field service, customers, oems)

II. Required Hardware

1. min config of hardware is required to test memory boards as follows:

multibus card cage, power supply

processor board

memory board(s)

terminal	one of :  sun video console (need video bd),

televideo-like terminal

boot device	one of :  local disk (need disk ctlr),

local tape (need tape ctlr),

remote disk via ethernet (need ethernet)

Note: this memory test runs out of ram and is not relocatable.  The

diag is loaded at location 0x4000 and occupies memory up to

0x9800.  So in the first meg of memory only addrs 0x9800-

0xfffff is tested.  For a board test station it is recommended

that a known good memory board is used and kept for the first

meg of memory, while the board under test should be configured

for the second (third and fourth) meg of memory.

2. memory board organization (cross ref karl's verbiage)

a. board 1 megabyte = 0x100000 = 1048576 bytes

b. 8 banks	@ 0x20000 = 131072 bytes (128K)

physical banks are named 0 - 7 as follows

bank 0 : locations U1200 - U1218 } banks 0-1 addrs 0x00000 - 0x3ffff

bank 1 : locations U1220 - U1238 }

bank 2 : locations U1300 - U1218 } banks 2-3 addrs 0x40000 - 0x7ffff

bank 3 : locations U1320 - U1238 }

bank 4 : locations U1400 - U1218 } banks 4-5 addrs 0x80000 - 0xbffff

bank 5 : locations U1420 - U1238 }

bank 6 : locations U1500 - U1218 } banks 6-7 addrs 0xc0000 - 0xfffff

bank 7 : locations U1520 - U1238 }

note that addresses within bank range don't stay within physical bank

because of a9, but switch back and forth between 2 banks every

0x200 bytes (eg addrs 0x000-0x1ff in bank 1, addrs 0x200-0x3ff

in bank 0, 0x400-0x5ff in bank 1, etc, addrs 0x40000-0x401ff

in bank 3, addrs 0x40200-0x403ff in bank 2, addrs

0x40400-0x405ff in bank 2, etc)

c. 18 64Kx1 chips in each bank for 16 bits (1 word or 2 bytes) + 2 parity bits

for upper/lower byte of each word

d. board select addr A20, A21, A22 compared with dip switch, memory board can

be configured for any one of the following address ranges:

0x000000 - 0x0fffff (switch 1 on, everything else off)

0x100000 - 0x1fffff (switch 2 on, everything else off)

0x200000 - 0x2fffff (switch 3 on, everything else off)

0x300000 - 0x3fffff (switch 4 on, everything else off)

0x400000 - 0x4fffff (switch 5 on, everything else off)

0x500000 - 0x5fffff (switch 6 on, everything else off)

0x600000 - 0x6fffff (switch 7 on, everything else off)

0x700000 - 0x7fffff (switch 8 on, everything else off)

III. Required Software

???

IV. Memory Diagnostic Operating Instructions

0. loading and starting

Turn on system, after power-on rom diags are run, system automatically

begins booting unix.  Break out of the boot and return to the rom

monitor by typing L1-a (hold down the L1 key while typing a) on the

sun2 console or by typing <break> on the televideo-type terminal.

At this point type k1 to the rom monitor to reset memory maps to

initial state.  We are now ready to boot the memory diagnostic.

Remember there are 3 ways of booting

a. from local disk

assuming the diagnostic 'mem.120.diag' exists on your local

disk in the /pub/stand (fileserver) or the /stand (standalone)

directory, the diagnostic is loaded by typing

> b stand/mem.120.diag

to the rom monitor prompt

b. from remote disk via ethernet

assuming the network fileserver has a partition reserved for

the system under test (ie it is a legitimate client) and that

the diagnostic exists in the /pub/stand on the fileserver, the

diag is loaded by typing

> b stand/mem.120.diag

to the rom monitor prompt

if the system under test is not a client of the fileserver

where the diagnostic lives in the /pub/stand directory, the

use the following boot command to the rom monitor

>b ec(,<fileserver_host_net_#>)stand/mem.120.diag

c. from tape (i've still got to verify this info)

assuming you have a tape with a bootable image of the memory

diagnostic on tape, use the following commands to the rom

monitor to load it.

b st()	<from scsi tape 1/4" cartridge>

b ar()	<from archive tape 1/4" cartridge>

b mt()	<from tape master 1/2" tape reel>

Note: No matter how the diagnostic is booted, once it is loaded, it

is automatically started.

1. menu

The menu is displayed whenever a single <cr> is typed at the

prompt (which is the first thing one sees after booting the diag).

Command :	(this is the prompt)

(This is so you're not forced to wade thru a menu if you know what

you're doing.)

The menu is displayed as follows (everything between dashed lines):

-----------------------------------------------------------------------------

Sun 120' Memory Test REV 1.1 7/31/84

Start of memory under test : 0xxxxx

Size  of memory under test : 0xxxxxx

Command selections:

A	set default address

S	set default size

f	fill memory

d	display memory

c	constant pattern test

r	random pattern test

a	address test

u	uniqueness test

x	checker pattern test

g	diagonal pattern test

m	byte, word, long mode

p	parity gen/chk on/off

w	wait on error

s	scopeloop on error

e	error message mode

l	loop

h	help

q	quit

Command :

-----------------------------------------------------------------------------

The following is displayed for "help". (type h at the command prompt)

(everything between the ==s) :

(Note the third column with the parameters required for each command indicated)

=============================================================================

A	set default address	A address

S	set default size	S size

f	fill memory          	f address size pattern

d	display memory       	d address size

c	constant pattern test	c address size pattern passcnt

r	random pattern test	r address size seed passcnt

a	address test         	a address size passcnt

u	uniqueness test		u address size incr passcnt

x	checker pattern test	x address size pattern passcnt

g	diagonal pattern test	g address size passcnt

m	byte, word, long mode	m [0|1|2]

p	parity gen/chk on/off	p [0|1|2|3]

w	wait on error		w [0|1]

s	scopeloop on error	s [0|1]

e	error message mode	e [0|1|2]

l	loop 			l loopcount

h	help			h

q	quit			q

=============================================================================

2. parameters

a. address b. size

All of the tools and the tests require address and size parameters.

(the address being the beginning address (low) of memory to be tested

and the size being the number of bytes in the block of memory to be

tested.) The address and size parameters are always specified in hex

notation. (eg 0x100000).  The size parameter always specifies a number

of bytes (no matter what the data mode - see further discussion under

modes - byte, word, long)).  If these parameters are not supplied (see

further discussion of techniques under the specials heading) then

the default address and size are used.  The default address and size

are the address and size printed at the head of the menu, set up

in the following ways:  1. at the beginning of the diag (right after

boot) the page boundary (pages are 2K = 0x800 bytes) following the

end of the diagnostic itself is taken to be the start address of

memory under test, while the size is taken to be the total memory

size of the system (determined and offered by the diag/mon/boot rom)

minus the start address. *or* 2. the addr and size parameters may

also be set using the commands 'A' and 'S'.

c. passcnt

the passcnt parameter specifies how many times to run through the

test, before continuing on to the next test specified on the command

line or before prompting for another command.  it is always specified

in decimal and it is always the last parameter supplied to the tests.

d. others

some of the tools and tests require an additional parameter that has

to do with the data used.  if present, these parameters are always

the 3rd paramter supplied.

pattern

the pattern parameter is the data to be written and read from

each memory location in the block of memory to be tested.  it

is always specified in hexadecimal notation.  the pattern

parameter is used in the following tools or utilities and

tests: fill, constant, checker

seed

the seed parameter is the seed used to generate the random

sequence of data for the random pattern test.  it is

always specified in decimal notation.

incr

the incr parameter is the increment used to generate a series

of numbers used in the uniqueness test.  the series is as

follows {n, 2n, 3n, 4n, ... } where n=incr.  it is always

specified in hexadecimal notation.

e. special notations

separator	';'

the separator is used between commands at the command line

prompt.  it must be isolated on the line (ie surrounded by

spaces).  it is this feature that allows tests to be

flexibly sequenced.

for example:

Command : c 8000 1000 aaaa 2 ; c 8000 1000 5555 2

default 	'.'

this symbol is used as a place holder to indicate default

values for some parameters while specifying others occurring

later in the command line.

for example:

Command : c . . aaaa 3

forever		'*'

forever is approximately infinty or 0xffffffff or

4294967295 (decimal) times.  use the forever symbol

for the passcnt parameter to loop on a single test

"forever".

for example:

Command : c 100000 100000 ffff *

null		<nothing>

null is simply nothingness if a parameter is not supplied

then the default values are used

for example:

Command : c 100000 100000

or simply

Command : c

Note : for more information on the default values see the section

on the specific test as well as the section on default

parameters below.

3. tools

a. fill			f addr size pattern

The fill command allows one to fill a specified block of memory with

a specified pattern.  Memory can be filled with bytes, words, or long

words of the given data pattern depending on the setting of the data

mode of the diagnostic (see the discussion of modes - byte, word, long

below).

example:

Command : f 80000 80000 0

Clears the 1/2 megabyte of memory at 0x80000 to 0.

defaults:

f daddr dsize ffffffff

b. display		d addr size

The display command allows one to display a specified block of memory

on the console or terminal.  Data is always (no matter what the data

mode) read and displayed byte at a time.  Each line of the display

contains the hexadecimal address (always a multiple of 0x10 except

for the first line if the specified block doesn't begin on a multiple

of 0x10 boundary) followed by 16 bytes of data grouped in long words

(by 4s).  While there is no paging of the display, ^s pauses the

command and display for inspection of the data.  After a ^s, any key

typed causes the command and display to continue.  Any key other than

^s interrupts the command and returns (the user) to the command line

prompt.

example:

Command : d 100000 20

100000:   00 01 02 03   04 05 06 07   08 09 0a 0b   0c 0d 0e 0f

100010:   10 11 12 13   14 15 16 17   18 19 1a 1b   1c 1d 1e 1f

Displays 32 bytes of data starting at 0x100000 as shown.

defaults:

d daddr dsize

4. tests

a. constant pattern	c addr size pattern passcnt

The constant pattern test tests the specified block of memory using the

specified data pattern.  First the memory block is filled with data,

then it is read back and compared with the specified data pattern.  If

the data read from an address location does not match the original

data pattern an error is flagged.  This test can be run in byte, word,

or long mode (see discussion under mode - byte, word, long).

example :

Command : c 100000 100000 aaaa 10

Tests the single megabyte of memory at 0x100000 using the data pattern

0xaaaa.  The test is repeated 10 times.

defaults:

c daddr dsize 55555555 1

b. random pattern	r addr size seed passcnt

The random pattern test tests the specified block of memory using a

sequence of random numbers generated from the specified seed.  The

random number generator is the same as that in the 'C' run-time

libraries.  First the block of memroy is filled with data, the

random sequence is reseeded, and the data is then read back and

compared with data that was originally written.  If the data read

from an address location does not match the original data pattern

and error is flagged.  This test can be run in byte, word, or long

mode (see discussion under mode - byte, word, long).

example:

Command : r 10000 400 3 5

Tests the 1k block of memory at 0x10000 using a random series of

numbers seeded by the number 3 as data.  The test is repeated 5 times.

defaults:

r daddr dsize 7 1

c. address		a addr size passcnt

The address test tests the specified block of memory using the low

order bits of the address of each location as data.  The number of

bits used depends on the data mode set up as this test can be run in

byte, word, or long mode (see discussion under mode - byte, word,

long).  There are two passes through the block of memory, the first

pass uses the lower address bits as is, the second pass uses the

complement of the lower address bits (see example for further

clarification).

example:

Command : a 100000 100000 1

Tests the 1m block of memory at 0x100000 using the low order bits

of the address of each location as data as well as the compliment

of those bits.  The test executed only one time.

For the sake of clarity, the series of numbers used as test data is

detailed below for this example for each of the modes:

byte

{00, 01, 02, 03, 04, ... } and

{ff, fe, fd, fc, fb, ... }

word

{0000, 0002, 0004, 0006, 0008, ... } and

{ffff, fffd, fffb, fff9, fff7, ... }

long

{00000000, 00000004, 00000008, 0000000c, ... } and

{ffffffff, fffffffb, fffffff7, fffffff3, ... }

defaults:

a daddr dsize 1

d. uniqueness		u addr size incr passcnt

The test for address uniqueness tests the specified block of memory

using the sequence {incr, 2 * incr, 3 * incr, 4 * incr, ... } for

the test data.  This test can be run in byte, word, or long mode

(see discussion under mode - byte, word, long).

example:

Command : u c0000 40000 5 100

Tests the 1/4m block of memory at 0xc0000 using the series

{5, a, f, 14, ... } as data.  The test repeated 100 times.

defaults:

u daddr dsize 1 1

e. checker		x addr size pattern passcnt

Checkertest writes patterns of pattern and pattern\ in a series of

write-read scans as follows:

Pass 0 writes every other word with the alternating patterns.

Pass 1 writes two words of each in sequence every four words.

Pass 2 writes four words of each in sequence every eight words.

And so on until the cell size is half the block size.

The checker test requires a number of write-read scans over the block

of memory under test (which explains why it takes so long to run!).

The data used is an alternating sequence of pattern and ~pattern (the

complement of pattern), and hence the name checker (short for checker

board).  The test scans are as follows:

the first scan writes alternating words of pattern and ~pattern

thruout the block of memory,

the second scan writes two words of pattern and two words of ~pattern

(ditto)

the third writes 4 words of pattern and 4 of ~pattern ""

the fourth 8 pattern then 8 ~pattern ""

and so on

the last scan will write size/2 words of pattern and size/2 words of

~pattern

(how do you explain that in words??)

(of course each write scan is followed by a read scan checking for the

pattern that was just written flagging errors where something else is

found)

the following equation applies:

size = 2 ^ n

where n is the number of scans required for this test

the size is assumed to be a power of 2

this test only runs in word mode

example:

Command : x 100000 100000 aaaa 10

Tests the 1m block of memory at 0x100000 using the patterns 0xaaaa

and 0x5555 in a checkerboard pattern as data.  The test repeated

10 times.

defaults:

x daddr dsize 5555 1

f. diagonal		g addr size passcnt

The moving diagonal test requires a number of write-read scans through

the entire memory bank.  On the first scan, all bits are written to

DATA\.  The read scan verifies the correct operation of the array

under these conditions.  On each succedding scan, the position of the

diagonal of DATA is shifted until, on the last scan, it has occupied

every possible position in the array.  Each cell has once been the

only DATA cell in a row and column of DATA\.

The diagonal test is actually a modified galpat test (hence the 'g'

to call up the command).  This test also requires a number of

write-read scans thru each bank (or row depending on karl's verbiage)

of the block of memory under test.  At the beginning of the test the

memory is initialized to 1's (every bit!).  Then the test proceeds in

the following sequence:

on the first scan a word of zeroes is written at particular locations

in memory causing a diagonal of 0's in each memory chip's memory array.

on each successive scan the diagonal is shifted until it has occupied

all of the diagonal positions of each memory chip's array.  in other

words each cell of the memory array has been the only 0 cell in a row

and column of the array.

each read scan checks for the diagonal of 0's in a field of 1's.

the size is assumed to be a multiple of 0x40000

this test only runs in word mode

example:

Command : g 100000 80000 1

Tests the 1/2m block of memory at 0x100000.  The test executed

only one time.

defaults:

g daddr dsize 1

5. modes

a. byte, word, long data mode

default : m 1

to set the mode to byte: m 0

to set the mode to word: m 1 (or just m)

to set the mode to long: m 2

applies to the following tests:

constant, random, address, uniqueness

also applies to the fill command

default is word (and is set to word mode on power up)

b. parity on/off

default : p 3

to turn parity on:  p 1 (or just p)

to turn parity off: p 0

default is on (also on power up)

c. errors : continue, wait or scopeloop

default : w 0

default : s 0

to wait (or stop) on error: w 1

no wait:                    w 0

to scopeloop on error: s 1

no scopeloop:          s 0

after on error has occurred with either of these modes on, the test

may be continued by typing any key.

scopeloop continually writes/reads the data causing the failure to

the address where the failure occurred (also taking into account the

byte, word, or long mode of the test)

with scopeloop on, when an error occurs (no matter what the error

message mode), the following is displayed:

.. looping

(<obs>)+ *or* (<obs>)-

where <obs> is the observed value, and the + message is printed when

the observed value matches the expected value and the - message is

printed when the observed and expected values don't match.

default is wait on error off (also on power up)

default is scopeloop on error off (also on power up)

d. error messages on or off

default : e 1

to print or display error messages: e 1

to turn off all message output:     e 0

6. loops

syntax : l loopcnt

default : l 4294967295

7. default parameters

The default parameters for each of the commands are summarized below:

Command : A daddr

Command : S dsize

Command : f daddr dsize ffffffff

Command : d daddr dsize

Command : c daddr dsize 55555555 1

Command : g daddr dsize 1

Command : x daddr dsize 5555 1

Command : r daddr dsize 7 1

Command : a daddr dsize 1

Command : u daddr dsize 1 1

Command : m 1

Command : p 3

Command : w 0

Command : s 0

Command : e 1

Command : l 4294967295

Where daddr and dsize above are the default address and size parameters.

The current default address and size parameters are always printed at

the head of the menu and are set in the following ways:

1. at the beginning of the diag (right after boot) the page boundary

(pages are 2K = 0x800 bytes) following the end of the diagnostic

itself is taken to be the default address parameter, while

the default size parameter is taken to be the total memory size of

the system (determined and offered by the diag/mon/boot rom) minus

the default address.

-or-

2. the default addr and size parameters may also be set using the

commands 'A' and 'S'.

(for more info see the section on parameters, address and size above)

8. default test sequencing

to be determined

V. Error Message Interpretation

Anomalies: Currently, if an error is detected, the memory location

where the error occurred is read twice (the first time when the error

is detected, the second time to pass the observed value to the

error handler).  The problem with this is that on the second read

the data read will occasionally match the expected value.

It is the second value read that is displayed in the error message.

This explains confusing error messages similar to the following:

constant pattern test failed @ 0x140000  exp (0x5555)  obs (0x5555)

1. bus errors

2. parity errors

3. addr exp obs

4. chip mapping

VI. Recommended Test Procedure

yet to be determined

1. testing the memory board circuits

2. p2bus interface

3. board select

4. bank select

5. memory array

6. parity