CHAPTER 5   SOME EXCLUSIVE FEATURES OF A86

The IF Statement

As a "nudge" in the direction of structured programming, A86

offers the IF statement.  Suppose you want to conditionally skip

around just one instruction.  Ordinarily, this would require, for

example:

JNZ >L1    ; skip the following move if NZ

MOV AX,BX  ; make this move only if Z

L1:          ; this label exists only for the above skip

You may replace the above code with the single line:

IF Z MOV AX,BX

The above line generates exactly the same code as the previous 3

lines-- a conditional jump of the opposite condition, around the

statement given in the tail of the IF statement.  The statement

can be a macro call, giving you the opportunity to skip something

more complicated.

You may use any condition that would follow the "J" in a

conditional jump instruction, except CXZ, which does not have a

reverse condition.  The assembler interprets the condition by

appending a "J" to the beginning of the condition; so that the

symbols "C", "NC", "Z", "NZ", etc. are not reserved by the

assembler, and can be defined in other contexts.

Multiple operands to PUSH, POP, INC, DEC

A86 will accept any number of register operands for the

instructions PUSH, POP, INC, and DEC; it will generate the

appropriate machine instruction for each operand.  For example,

the statement PUSH AX,BX is the same as the two statements PUSH

AX and PUSH BX.

A numeric operand appearing in an INC or DEC statement will cause

the previous INC(s) or DEC(s) to be propagated that number of

times.  For example, the statement INC AX,4 will generate 4 INC

AX instructions.  The statement DEC AL,BX,2 will generate DEC AL,

DEC BX, DEC AL, DEC BX.  Sorry, numeric operands are not allowed

if any of the operands affected was a forward reference or

relocatable quantity; e.g., INC FOO,2 where FOO is undefined.  In

most such cases, you'll want to code the more efficient ADD FOO,2

anyway.

5-2

Conditional Return Instructions

Programmers accustomed to the conditional return instructions of

the 8080/Z80 will appreciate the following feature: A86 allows

the operand to a conditional jump instruction to be one of the

three RET instructions RET, RETF, or IRET.  The assembler will

find a nearby return instruction of the indicated flavor, and use

that as the target for the conditional jump.  For example, JZ RET

is the replacement for the 8080's RZ return-if-zero instruction.

In other 8086 assembly languages, you have to find the nearby

instruction yourself, attach a label to it, and use that label.

Note that it does not suffice to attach a label to a single RET

instruction and use that label throughout the program: the range

of conditional jumps is only 128 bytes in either direction.

What happens if A86 does not find a nearby return instruction? In

that case, A86 issues an error, "02 Jump > 128", for the next

matching return instruction in the program.  If there is no

subsequent return instruction, the return mnemonic will appear as

an undefined symbol at the end of the program.   In either case,

you correct the problem by inserting a free-standing return

instruction at some nearby point in the program, where it will

not affect the existing code (typically following an

unconditional JMP instruction).  If there is no good place to

insert a return instruction, you can always replace the "Jcond

RET" with an "IF cond RET".

A86 extensions to the MOV and XCHG instructions

There are a number of MOV and XCHG instructions available in A86

that are not a part of the machine instruction set.

First, moves between segment registers, and of immediate

constants into segment registers are allowed.  For example, if

you code MOV ES,DS , the assembler will generate a PUSH DS

followed by a POP ES; which will effect the move that you

intended.   If you code MOV DS,0 , the assembler will generate

PUSH AX; MOV AX,0; MOV DS,AX; POP AX.  This is mainly a

convenience for D86 users to load segment registers manually.

Second, MOV allows 3 operands.  A statement MOV x,y,z is

equivalent to the two statements MOV y,z followed by MOV x,y.

Sorry, but segment overrides are not allowed in conjunction with

3-operand MOVs.  The override preceding the MOV is ambiguous in

its meaning; and overrides within operands cannot be handled

correctly by A86.  You'll have to code two MOV instructions if

you want either or both to have a segment override.

Third, A86 accepts a MOV of a word-sized memory operand into

another word-sized memory operand.  A86 handles this the same way

it handles a MOV of segment registers: it generates a PUSH of the

source followed by a POP of the destination.

5-3

Finally, A86 allows the XCHG of a segment register (except CS)

with any other word-sized quantity, as well as the XCHG of two

word-sized memory quantities.  If there is no machine instruction

available for XCHG a,b, then A86 generates PUSH a followed by MOV

a,b followed by POP b.

Local Labels

If you examine most assembly language program symbol tables, you

will find that the symbols can be partitioned into two levels of

significance.   About half the symbols are the names of

procedures and variables having global significance.  If the

names of these symbols are chosen intelligently and carefully,

the program's readability improves drastically. (They usually

aren't chosen well, most often because the assembler restricts

symbols to 6 letters, or because the programmer's habits are

influenced by such assemblers.)

The other half of the symbols in a program have a much lower,

local significance.  They are only place markers used to

implement small loops and local branching (e.g., "skip the next 2

instructions if the Z-flag is set").  Assigning full-blown names

to these labels reduces the readability of your program in two

ways:  First, it is harder to recognize local jumps for what they

are-- they are usually the assembly language equivalent of high

level language constructs like IF statements and WHILE loops.

Second, it is harder to follow the global, significant symbols

because they are buried in a sea of the place marker symbols in

the symbol table.

A86 solves this problem with local symbols.  If a symbol in your

program consists of a single letter followed by one or more

decimal digits (L3, X123, Y37, etc.), then the symbol is a local

symbol.  Local symbols do not appear in the A86 XREF

cross-reference listing. They can also be redefined to something

completely different later in the program.

Because local labels can be redefined, you must take care to

specify which one you are referring to in your program.  If your

reference is a forward reference (the label occurs further down

in the program from the reference), then the reference must be

preceded by a ">".  For example,

L2:

MOVSB

INC BX

LOOP L2    ; lack of ">" means L2 is above this statement

.

.

JNZ >L2    ; ">" indicates L2 is below this statement

.

.

L2:

5-4

I recommend that you assign all your local labels the names L0

through L9.  If your program is so complex that it needs more

than 10 place holders in any one stretch of code, then that

stretch needs to be rewritten.

Operands to AAM and AAD Instructions

Those of you who have examined 86 family opcodes with an eagle

eye will have noticed a somewhat spurious "0A" opcode generated

after every AAM or AAD instruction.  The opcode is there to

provide the constant divisor or multiplicand for the instruction.

Believe it or not, there wasn't enough room in the microcode of

the original 8086 to hold this constant!  Although Intel has

never announced the generality of AAM and AAD, it is there: you

can substitute any other constant for 0A (decimal 10), and that

constant will be used.  A86 supports this by letting you give a

constant byte-sized operand to AAM or AAD.  Particularly useful

are the instructions AAM 16, which unpacks AL into nibbles AH and

AL; and AAD 16, which reverses the process, packing nibbles AH

and AL into AL.

WARNING: A couple of my users point out to me that the AAD

instruction with a general operand won't work on the NEC V20 and

V30 chips.  The operand is assumed to be 10 no matter what it

really is.  Since a large number of PC "speed up" kits involve

switching to NEC chips, this will be seen on many PC's.  You

should not use AAD with an operand if you want your program to

run on everybody's machine.  Too bad.  AAM works fine, though.

Single-Operand Forms of the TEST Instruction

A86 allows the TEST instruction to have a single operand, to set

the flags according to the value of the operand.  If the operand

is a register, A86 generates a TEST of the register with itself.

If the operand is a memory quantity, A86 generates a TEST of the

memory with the constant -1 (i.e., the quantity will be ANDed

with an all 1's constant).  For example, instead of TEST DL,DL,

you can code simply TEST DL.  Instead of TEST WVAR,0FFFF, you can

code simply TEST WVAR.