DOCUMENTATION FOR FILES RAMDRIV.ASM AND RAMLD.ASM

by Herb Shore       Dec. 27, 1982

These programs are used to make extra memory beyond the usual

64K function as though it were a very fast disk drive.  The extra

drive (drive C: in my system) can be used in the same way as all the

other drives, though of course all data on this drive is lost when

the power is turned off.  The way I use the system is to PIP over my

working files to drive C: after activating the RAMDRIV, working on

the files while logged onto C:, and then PIPing them back to my

floppy before shutting down the computer.  For repetitive operations

such as assembling, compiling, using a word-processor with overlay

files, etc. the increase in speed is fantastic.

The system is especially easy to use if you have replaced the

standard CCP with ZCPR.  With ZCPR, the standard utility files (PIP,

STAT, MAC, etc.) can remain on drive A:, since these files are not

accessed after they have been loaded.  ZCPR will automatically

search A: for these files if they are not present on C:.  Only those

files which are repeatedly accessed have to be on the fast drive.

To implement the system, prepare file RAMDRIV.COM using the

information on files RAMDRIV.ASM and RAMLD.ASM.  Then the RAM drive

is activated by executing RAMDRIV.COM.  This program will ask if you

wish to clear the directory area of C:.  If the computer has just

been turned on, answer YES, since the directory area contains

garbage.  However, if your system has crashed due to a program

runaway, it is very likely that extended memory is still OK.  In

that case you can do a cold boot, run RAMDRIV, and answer NO when

asked if the directory should be cleared.

SOFTWARE REQUIREMENTS:  The RAMDRIV system requires 1K of

global RAM above the BIOS.  This may mean you have to reduce the

size of your CP/M system (I use a 63K CP/M).  Other than providing

this space, there are no special requirements for the size of the

CP/M system or the location of the BIOS; this is determined by the

program.

HARDWARE REQUIREMENTS:  At least 1K of global RAM must be

provided.  Global RAM is memory that responds to the standard 16 bit

address bus independently of the setting of the 8 bit extended

address bus.  If your global RAM is larger than 1K, the system will

still work, but the memory available for the extra disk is reduced.

The system must provide a way of setting the extended address bits

by outputting the extended address to an output port.  If your

present system does not have this capability, it can be easily

implemented.  Turning 1K of RAM into global RAM is also not

difficult.  One way is as follows: connect a 6-input AND gate to

monitor address lines A10-A15.  When all six lines are high, the

upper 1K of memory is being addressed.  When this happens, tri-state

the circuit (output port) driving the extended address bus and force

all 8 bits of this bus to be low.  Thus, the RAM at extended address

00H will become global for the upper 1K of the 64K address space.  A

word of caution: If your system uses DMA devices, these devices may

also attempt to drive the extended address bus.  Make sure that any

extra circuitry you add is disabled during DMA operations in order

to avoid bus conflicts.

#dress bus.  Make sure that any

extra circuitry you add is disabled during DMA oper