EPRINT - A multiple file printing program for the Epson Printers

Copyright 1982 by T. Shapin.  Permission to reproduce and dis-

tribute without charge is granted.

The Epson printers allow a variety of type fonts controlled by

escape sequences and control characters that are sent to the

printer along with the text to be printed.  On the other hand,

backspaces are normally ignored by Epson, but this program handles

backspaces in software.  Tab stops are set every eight spaces so

tabs imbedded in the text will move to the next stop.

This program contains the subroutine written by Gary P. Novosielski

and published in Dr Dobbs Journal, December 1981, and code from

MLIST, originally written by Keith Peterson.

The program is run as a CP/M program by typing:

EPRINT [drive:]<filename.type> [F]

where   < > = required, [ ] = optional

Ambiguous file names may be used (i.e. EPRINT *.ASM will print all

files with "ASM" extensions).

An optional "F" following the file name will cause a formfeed to

be sent to the printer after the end of each file.

The Epson printer is initialized by flushing any characters in its

buffer, setting horizontal tab stops, and turning off any special

fonts.

From then on, imbedded control codes will control special font

features.

Double width characters are turned on with a control-N and turned

off at the end of a line or with a control-T.

Tabs are the standard control-I character but seem to be ignored

if double width mode is on.

Compressed printing is enabled if a control-O is present anywhere

in a line.  (Remember, the whole line is stored in a buffer and

printed only when a carriage return or line feeed is sent.)  This

mode will continue until a line containing a control-R is sent.

Emphasized printing is set on by a line containing an Escape "E"

sequence.  This causes all characters to have twice as many dots

in the horizontal direction.  It will continue until a line with

an Escape "F" sequence in it is sent.

The double strike mode in turned on if a line contains an Escape

"G" sequence.  This causes each line to be printed twice, once

normally, and the second time after a one-dot height line advance.

This mode is turned off when a line containing an Escape "H" is

sent.

Underline is handled in software because the printer ignores

backspace characters.  If the string "ABC///___" is sent (where

the backspace code, control-H is represented by "/"), this

program will print the "ABC" with one motion of the print head;

then return the carriage and send three spaces followed by the

three underlines to the printer.

For the purpose of printing emphasized or double strike letters

within part of a line, this program uses the control-W code to

cause the line as transmitted so far to be printed according to

the current setting of the mode flags but does not reset the

column counter to zero.  For example to write "The middle word

is emphasized" and print "WORD" in emphasized style, you would

send "The middle" control-W escape E "word" control-W escape F

"is emphasized".

Filenames are displayed on the console as they are printed. A

file can be skipped by typing control-X while the program is

running.  A control-C will abort the program and return to the

operating system.  Certain files such as ".OBJ" and ".COM"

file types will not be printed.

$#is causes all characters to have twice as many dots