Date: 21 Feb 1981 1544-PST

From: Jorge Phillips

To:   Northstar Users

Re:   Split-speed modems and N*'s

I have recently modified my Horizon to run a split-speed modem

(Prentice 1200/150) on serial port 2.

MAKING PORT 2 BE SPLIT SPEED (1200/150)

1. Locate headers at 2D and 4D in motherboard

2. Modify 2D (baud-rate) header as follows:

2.a Remove wires originating at pins 5 and 6.

2.b Solder wire from pin 5 to pin 9 (150 transmit).

2.c If jumper J4 exists in your motherboard near 5D (Look

for a wire there)  remove it (since  pin 9 of  header

would be jumped  to 75 baud)  and restore  electrical

path from  pin 9  of 2D  header to  pin 10  of  LS393

counter at  5D.  The standard  motherboard  has  this

electrical path on the PC board (i.e. default=150 baud).

2.d Solder wire from pin 6 to pin 13 (1200 recieve)

3. Modify 4D (port 2 configuration) header as follows:

3.a Get a 16 pin dip header.

3.b Make port be a modem port as follows:

solder wires between pins 1-16,2-12,3-14,4-15,5-13,7-10,8-9

3.c Replace old by new header

and you are done!  hook up your modem with an RS232 cable and you

are ready to use MBOOT, MODEM2 or other suitable programs.

#################